iON Performance Stage 1 FMIC Installation Instructions

Congrats on purchasing our stage 1 FMIC kit for the MazdaSpeed Protegé.  The following is a brief installation guide, which will help you with the installation of the kit.  Please have the kit installed at a local shop, which has the proper equipment for the job. (call ahead of time to see if they have the ability to cut sheet metal).

Step 1:	Put the car on a hoist or on 4 jack stands.  Let the car cool for ~30mins.  Gather 	all necessary tools.

Step 2: Remove all the plastic plumbing from the turbo to stock I/C, and from the stock 	I/C to the throttle body.  Remember to be careful when taking off the stock by-	pass valve (BPV) Also remember to remove the airbox & all it’s plumbing this 	will make more room, making the install easier.

Step 3:	 Remove the front bumper, there are several screws/bolts at the top of the 			grill.  Using a razor, make the following trimming to the L&R bumper 			tabs on the lower airdam.  Set bumper cover aside now.

Step 4:	 Re-locate the power steering lines to behind the bracket.

Step 5: Mount the intercooler.  There are brackets underneath, as well as one bracket on 	the back.  Fit on a 2.5”to 2” 90 degree silicone hose onto the passenger side of the 	vehicle.  Mark the position of where the hose reaches the chassis.  Mark area with 	a marker or grease pen.  Make the tracing slightly larger then the hose to allow for 	movement.  Take off the silicone hose after your mark the position.

Step 6: Disconnect the power steering cooling lines behind, or have someone move them 	out of the way when cutting.  The hole to cut should look something like this.

Step 7: After hole is cut, use something to slightly sand down the edges to prevent injury.  	To inhibit corrosion, block off area and spray with some rust inhibiting paint.  	This will cover the edges and make sure there’s no rust that’ll develop here from 	the cutting.

Step 8:	Re-fit the 2.5” to 2” reducer 90 degree silicone elbow onto the intake side of the 	intercooler.  Fit a band clamp on the I/C end, don’t tighten, slip one over the pipe 	too.  From the inside of the engine bay, fit on a semi “J-shaped” pipe. This pipe is 	2” in diameter. On the end of this pipe, fit a 2” 90 degree elbow pointing up.

Step 9:	Fit the long 2” L-shaped pipe on the end of the 90 degree silicone elbow.  Remember to put the band clamps on loosely.  To the other end you will have to insert the last 2” 90 degree silicone elbow.  The free end will be attached to the compressor discharge on the turbo.

Step 10: Make sure that the band clamp here is facing in this specific direction, otherwise it may come in contact with the fan.

Step 11: Now you can go back and tighten all those band clamps.  Since you’re complete 	the “hot” side of things, we now move to the “cold” side of things.

Step 12: On the driver’s side of the car place a 45 degree silicone elbow onto the outlet of 	the endtank.  To the end of the 45 degree elbow attach the following “U-shaped” 	pipe.

Step 13: Mark the location of where the pipe will enter the engine bay.  Take off the pipe, 	and again carefully make the cut through the sheet metal.  Make the hole just 	slightly larger so that you can adjust the pipe.  Again, after making the cut, 	smooth out the edges with either sand paper or a sanding wheel.  You can apply 	some rust inhibitor paint here as well.

Step 14: Now slip on a band clamp to the 45 degree silicone hose attached to the 	intercooler.  Insert the U shaped pipe again (short side to the I/C), add another 	band clamp there.  Don’t tighten!  Inside of the engine bay should look like this:

Step 15: Slip on another 2.5” 90 degree silicone hose on the end of this pipe.  To this, 	connect the short pipe with the bung on it.  The fitting that comes in the box will 	be screwed into the bung.  Leave this loose so it can be positioned later on.

Step 16:  To the end of this pipe that was added in the last step, slip on the hump hose and 	connect the last pipe that connects the last silicone elbow to the throttle body.

Step 17: You can go back and now tighten all the band clamps.  If you don’t have the 	optional CAI, you can stop here and place back your stock air-box as well as it’s 	associated plumbing. (If you have the CAI upgrade, stop reading here and skip to 	step 17.  Adjust the fitting that was screwed into the bung, and attach the BPV to 	this.  Once things are all tightened, you can fit back the bumper cover and you’re 	ready to go!

Step 18: You’ll only need to follow these instructions if you have the optional CAI 	upgrade as well.  Since you have the stock air-box & it’s associated plumbing out, 	you’ll also need to undo the coolant overflow bottle. (Bracket is supplied for the 	relocation).

As you can see here, the overflow bottle is re-located beneath the new CAI (this is the fully assembled picture).

Step 19: Using the brackets supplied, first move the overflow bottle down into it’s new position.  There’ll be a 3” L-shaped pipe which will be used to connect the filter from the fender well to the MAF.

Step 20: Using WD40, spray liberally onto the compressor inlet as well as the reducing silicone coupler.  You will be re-using the stock band clamp here, so slip this over the hose before attempting to fit it over the turbo compressor inlet.  After that’s connected, slip the other t-camp onto the silicone hose and then using the pipe that’s “s-shaped” slip into the hose coupler.  Please make sure the fitting is at the top as in the picture below.

Step 21: At this point, you should connect the stock BPV into position.  Using the supplied hose to attach to the BPV, slip this onto the fitting on the CAI, and the other end into the I/C to throttle body charge pipe fitting.  Using the supplied hoses, connect the MAF up from both sides.  You’re now ready to bolt back your bumper etc.